# Tab 1

Swaninathan et al. (2003) A perspective on distortions

Baker & Nayar (1999) A Theory of Single-Viewpoint Catadioptric Image Formation

Bogner (1995) An Introduction to Panospheric Imaging

Ward (1965) Optical Properties and Uses of the Conical Mirror

Yagi et al. (1994) Real-time omnidirectional image sensor (COPIS) for vision-guided navigation

Lin & Bajcsy (2006) Single-viewpoint, catadioptric cone mirror omnidirectional imaging theory and analysis

Lin & Bajcsy (2001) True single view point cone mirror omni-directional catadioptric system

Baker & Nayar (1999) A Theory of Single-Viewpoint Catadioptric Image Formation

- SVP is the requirement for catadioptric sensors; this paper describes all of the solutions(rotated conic sections).

- Derived fixed SVP constraint equation, solutions are planar, conical, spherical, ellipsoidal, and hyperboloidal.

- For conical mirrors, the pinhole of the camera must be at the apex of the cone, which is thus a degenerate solution.

- The circular locus lies on the cone only if the cone tip angle is 60 degrees.

- Yagi et al. (1994) and Bogner (1995).

- Derived the resolution mapping equation for hyperboloidal and ellipsoidal mirrors, should apply to conical mirrors but might not be very robust (worth giving it a try).

- Blurring analysis for hyperboloidal and ellipsoidal mirrors.

Yagi et al. (1994) Real-time omnidirectional image sensor (COPIS) for vision-guided navigation

- Conical mirror + TV camera aligned with the cone’s axis.

- Small aperture and short focal length lens.

- Installed on the robot for collision detection.

- Use trajectories of vertical edges over time as well as the azimuth direction(the angle of the robot towards the object), to compute the direction of relative motion.

- All computations in polar coordinates, no unwarping.

Bogner (1995) An Introduction to Panospheric Imaging

- They aim to capture a spherical field of view (360° horizontal + ≥ 180° vertical).

- Fisheye lenses + spherical/conical mirrors.

- Image Transformation Engine: LUT for pixel address imaging.

Lin & Bajcsy (2006) Single-viewpoint, catadioptric cone mirror omnidirectional imaging theory and analysis

- Detailed notes in another file.

- They proved that a conical mirror can still create SVP, which contradicts Baker & Nayar (1999).

- They provided the experiment setup in Lin & Bajcsy (2001).

Swaninathan et al. (2003) A perspective on distortions

- SVP (and approximate) can be undistorted perfectly, including a conical mirror.

- In terms of undistortion algorithms, they only derived ones for MVI (Multi-Viewpoint Image).

- Undistorting MVI requires a simple geometry primitive, with a basic uncertainty model for parameters.

- Proposed a metric to quantify distortions relative to a true perspective view.

Ward (1965) Optical Properties and Uses of the Conical Mirror

- Concave conical mirror.

- Analysis suggests not strictly SVP, but can approximate it in certain configurations.

- Derived relation between cone angle and magnification, focused on the geometric reference at the design level.

Lin & Bajcsy (2001) True single view point cone mirror omni-directional catadioptric system

- Provided the experiment setup in Fig 8, with a m12 s-mount lens.

- Provided a detailed unwarping algorithm in section 4.

- Removing a small amount from the tip does not affect the geometry, but only slightly affects the brightness.