- Conical mirror

  - Top angle approx. 90-110

- Camera

  - Fixed aperture for Logi C920

  - Need to disable auto focus with Logitech capture

- Position

  - Need pixel focal length, already retrieved with calibrateCamera() from opencv

  - Might not get the datasheet for CMOS

(Optional Upgrade)

Achromatic doublet (telescope)

Better SVP alignment

Analyze the Conical mirror aberration

Attach a thin plano-concave lens to the CCD

Attach a thick meniscus lens near the imaging lens

Calculations

- Vertical FOV of the camera

- Horizontal FOV of the camera

5. Minimizing Aberration Effects

- Change F/Number: Best at F/16 (at diffraction limit)

- Move the aperture: The farther away from the lens, the better. Restricted by vignetting.

Best at F/16 with 4.865 backwards.

- Change mirror shapes: The smaller the tip angle, the better. Needs to consider FOV.

- Change focal length: The shorter the f, the better.

- Change mirror size: Doesn’t matter. Just make sure sufficient FOV.

6. Frequency Response Analysis

7. Experiments

- SVP conditions are much more robust to longitudinal errors than lateral errors.

Can move up and down on the optical axis direction, but must be aligned with the tip.

- F/2 works badly, F/10 works great.

- A standard digital camera works well at 1/30s exposure.